Mechanics of Nanoscale Soft Contacts

Debashish Das1, Ioannis Chasiotis1

1Aerospace Engineering, University of Illinois at Urbana Champaign, 104 S Wright St, Urbana, IL, USA

The interaction between adhesion and friction at micro/nanometer length scales determines the macroscopic behavior of natural and synthetic materials, such as connective tissues, filtration systems, etc. In this work, micromachined MEMS-based force sensors were used to measure the rate-dependent interaction between polymer fibers in the diameter range of 400 nm–4 μm. Contact and fracture-mechanics models were used to estimate the contact area and explain observed jump instabilities. The interfacial shear strength during sliding was found to be constant for a broad range of contact radii. Shear yielding during sliding was identified as the origin of static friction in nanoscale soft contacts.